Trên  Router:

Switch – Router:

Router – Cloud:

Switch – PC1:

Router (1,123):

management network from the above interface summary: GigabitEthernet0/0

IP address for this interface: 192.168.1.1

Subnet mask for this interface [255.255.255.0] :

[0] Go to the IOS command prompt without saving this config.

[1] Return back to the setup without saving this config.

[2] Save this configuration to nvram and exit.

Enter your selection [2]: 2

configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface g0/0

Router(config-if)#description Gateway cho mang noi bo

Router(config-if)#ip address 192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface g0/1

Router(config-if)#description Noi len Cloud

Router(config-if)#ip address 10.0.0.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router#write memory

Router#show ip interface brief

Trên Switch:

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#interface fa0/1

Switch(config-if)#switchport mode access

Switch(config-if)#switchport port-security

Switch(config-if)#switchport port-security maximum 1

Switch(config-if)#switchport port-security mac-address sticky

Switch(config-if)#switchport port-security violation shutdown

Switch(config-if)#exit

Switch(config)#exit

Switch#

Switch#show port-security address

Switch#show port-security interface fa0/1

IP:

PC0: 192.168.1.2 255.255.255.0, subnet mask: 192.168.1.1

PC1: 192.168.1.3 255.255.255.0, subnet mask: 192.168.1.1

Lab 5:

Fix

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#ip dhcp snooping

Switch(config)#ip dhcp snooping vlan 123

Switch(config)#interface fa0/3

Switch(config-if)#ip dhcp snooping trust

Switch(config-if)#ip arp inspection vlan 123

Switch(config)#interface fa0/3

Switch(config-if)#ip arp inspection trust

Switch(config-if)#exit

Switch(config)#exit

Switch#